M5StampPLC

Batch Fill Controller

User Manual

Version 0.05

January 2026

# Table of Contents

1. What This Device Does

2. Understanding the Display

3. Basic Operation

4. Buttons and Navigation

5. Changing Settings

6. Cycle Counters (NEW)

7. Troubleshooting

8. Specifications

9. Maintenance

10. Safety Warnings

# 1. What This Device Does

The Batch Fill Controller automatically fills containers to a precise volume (in milliliters). It can control up to 4 filling stations at the same time, each working independently.

## How It Works:

- Press the start button (or flip the input switch)

- The valve/pump opens and liquid flows

- A flowmeter counts the liquid

- When the target volume is reached, the valve/pump closes automatically

- Release the start button to reset for the next fill

- The cycle counter increments (tracks total completed fills)

# 2. Understanding the Display

## Main Screen Layout

Example Display:

| CH | IN | OUT | ml | STATE | CYC |
| --- | --- | --- | --- | --- | --- |
| 1 | ● | ● | 52.3 | RUN | 15 |
| 2 | ● | ● | 0.0 | IDLE | 7 |
| 3 | ● | ● | 100.0 | DONE | 142 |
| 4 | ● | ● | 34.5 | FAIL | 23 |

### Header Information

- SP:100 ml - Setpoint (target fill amount)

- v0.05 - Software version

### Column Descriptions

- CH - Channel number (1-4)

- IN - Input button status: ● Green = ON, ● Red = OFF

- OUT - Output valve/pump: ● Green = Open, ● Red = Closed

- ml - Current volume dispensed

- STATE - Operating status (see below)

- CYC - Total completed fills (NEW in v0.05)

### Operating States

- IDLE (White) - Ready, waiting for start button

- RUN (Green) - Currently filling

- DONE (Yellow) - Fill complete, release button to reset

- FAIL (Red) - Fill canceled (button released early)

# 3. Basic Operation

## Starting a Fill

- Make sure the channel shows IDLE

  - If it shows FAIL, press and release the start button to reset

- Press and HOLD the start button (or turn input ON)

  - The IN indicator turns green

  - The OUT indicator turns green

  - The state changes to RUN

  - The ml counter starts increasing

- Keep holding the button until filling is complete

  - When the target volume is reached:

  - • OUT turns red (valve closes)

  - • State changes to DONE (yellow)

  - • The exact amount dispensed is shown

  - • Cycle counter increments by 1

- Release the start button

  - State changes back to IDLE

  - Counter resets to 0.0

  - Ready for next fill

## If You Release the Button Early

- The fill will stop immediately

- State changes to FAIL (red)

- The partial amount is shown

- Cycle counter does NOT increment

- To restart: Release the button completely, then press and hold again

# 4. Buttons and Navigation

The M5StampPLC has 3 buttons: [A]  [B]  [C]

## From Main Screen

- Button C - Enter Settings Menu

  - ⚠️ Only works when ALL channels show IDLE

## From Settings Menu

- Button A - Move up in menu

- Button B - Move down in menu

- Button C - Select highlighted item

- Hold Button C (1 second) - Exit back to main screen

# 5. Changing Settings

## Settings Menu Options

- Setpoint (all CH) - Adjusts batch size for all channels

- K-Factor CH1-4 - Calibration for each flowmeter

- Reset Cycle Counts - Resets all cycle counters to zero (NEW)

## Changing the Fill Amount (Setpoint)

- Wait for all channels to show IDLE

- Press Button C to enter Settings Menu

- Press Button C to select Setpoint (already highlighted)

- Press Button A to increase by 5ml

- Press Button B to decrease by 5ml

- Press Button C to save (returns to menu)

- Hold Button C for 1 second to exit to main screen

Range: 50-500 ml, Step: 5 ml

## Adjusting K-Factor (Advanced)

⚠️ Only adjust if your fills are consistently too much or too little.

The K-Factor tells the controller how many pulses from the flowmeter equal 1 milliliter of liquid.

- Higher K-Factor = More pulses needed = Fills LESS liquid

- Lower K-Factor = Fewer pulses needed = Fills MORE liquid

To Adjust:

- Enter Settings Menu (Button C)

- Use Buttons A/B to highlight the K-Factor for your channel

- Press Button C to select

- Use Button A (+0.1) or Button B (-0.1) to adjust

- Press Button C to save

- Test with a measuring cup and adjust if needed

Default K-Factor: 10.00 pulses/ml, Range: 0.1 to 100.0

# 6. Cycle Counters (NEW in v0.05)

The cycle counter tracks the total number of successful fills completed on each channel. This information is saved permanently and persists even after power loss.

## What the Cycle Counter Shows

- Location: CYC column on main screen (magenta color)

- Increments: Only when fill completes successfully (reaches DONE state)

- Does NOT increment: When fill is canceled (FAIL state)

- Storage: Saved to non-volatile memory immediately after each successful fill

- Display format: Shows actual number (142) or thousands (2k for 2,347)

## Resetting Cycle Counters

Important: Resetting counters will reset ALL 4 channels to zero at once.

Steps to Reset:

- Wait for all channels to show IDLE

- Press Button C to enter Settings Menu

- Use Button B to navigate down to "Reset Cycle Counts"

- Press Button C to select

- The screen will show current counts for all channels

- Press Button A for YES to confirm and reset all counters to 0

- Press Button B or C for NO/CANCEL to return to menu without resetting

## Use Cases for Cycle Counters

- Production Tracking: Monitor total production per station

- Quality Control: Verify production quantities at end of shift

- Maintenance Planning: Schedule valve/flowmeter maintenance based on cycle count

- Performance Analytics: Compare productivity across stations

## When to Reset Counters

- At the start of a new shift or production run

- After maintenance or calibration

- When changing products or batch sizes

- For weekly/monthly production reporting

# 7. Troubleshooting

| Problem | Solution |
| --- | --- |
| "Wait for IDLE" message appears | Trying to enter settings while a channel is running. Wait for all fills to complete. |
| Fill stops immediately | Not holding the start button. Press AND HOLD the button during entire fill. |
| Fill amount is wrong | K-Factor needs calibration. Measure actual amount and adjust K-Factor accordingly. |
| Channel stuck in FAIL | Button released during fill. Press and release button to reset to IDLE. |
| Cycle counter not incrementing | Counter only increments on successful completion (DONE state). FAIL state does not increment counter. |

# 8. Specifications

| Feature | Specification |
| --- | --- |
| Channels | 4 independent |
| Fill Range | 50 - 500 ml |
| Accuracy | Depends on flowmeter (typically ±1%) |
| Inputs | 8 digital inputs (4 used for start buttons) |
| Outputs | 4 relay outputs for valves/pumps |
| Flowmeters | 4 pulse counters (GPIO 1, 2, 4, 5) |
| Display | Color LCD with cycle counter display |
| Power | 24V DC |
| Communication | USB (Serial) + GPIO40/41 (Serial2) |
| Cycle Counter Range | 0 to 4,294,967,295 (32-bit) |
| Memory Storage | Non-volatile (settings and cycle counts retained during power loss) |

# 9. Maintenance

## Weekly Maintenance

- Clean the display with a soft, dry cloth

- Check all connections are secure

- Verify flowmeters are clean and free of debris

- Record cycle counts for tracking

## Monthly Maintenance

- Test fill accuracy with measuring cup

- Recalibrate K-Factors if needed

- Check valve operation

- Review cycle counters for maintenance scheduling

## As Needed

- Replace flowmeter if readings become erratic

- Clean or replace valves if sticking

- Update software if new version available

# 10. Safety Warnings

## ⚠️ ELECTRICAL SAFETY

- Only qualified personnel should install or service this device

- Disconnect power before opening enclosure

- Observe proper electrical safety procedures

## ⚠️ LIQUID SAFETY

- Use appropriate materials for your liquid type

- Ensure proper ventilation for volatile liquids

- Follow all safety data sheets for chemicals

## ⚠️ OPERATION

- Do not adjust settings while filling is in progress

- Do not force buttons

- Keep liquid away from electronics

# Quick Reference Card

## Normal Operation

- Wait for IDLE

- Press and HOLD start button

- Wait for DONE

- Release button

- Repeat

## Change Fill Amount

- Button C → Settings

- Select "Setpoint"

- Button A/B to adjust

- Button C to save

- Hold Button C to exit

## Reset Cycle Counters

- Button C → Settings

- Navigate to "Reset Cycle Counts"

- Button C to select

- Button A for YES (confirm)

## Reset After FAIL

- Release start button completely

- Press and hold to start new fill

# Notes

Use this space to record your specific settings:

| Channel | Product | K-Factor |
| --- | --- | --- |
| Channel 1: |  |  |
| Channel 2: |  |  |
| Channel 3: |  |  |
| Channel 4: |  |  |

Calibration Date: _______________

Next Calibration Due: _______________

Manual Version: 2.0  •  Software Version: 0.05  •  January 2026